BERKSHIRE LAKES
ARCHITECTURAL
REVIEW BOARD
STANDARDS

August 2024

Section		      Topic							Page

1			ARB Authorization						     2

2			ARB Alteration Request Process				     2

3			Definitions							     3

4			Contractor Responsibilities					     3

5			Drainage and Grading						     4

6			Landscaping  						   	     5

6			Landscaping Adjacent to Common Properties			     5

7			Fences								     6

8			Hedges								     6

9			Walls								     7

10			Shoreline							     7

11			Mechanical Equipment					     8

12			Accessory Structures						     8

13			Swimming Pools, Spas and Hot Tubs				     8

14		     	Lanai and/or Patio Roof					     8

15			Painting							     8

16			Driveways							     9

17			Roof Materials							     9

18			Garage Door Screens						     9

19			Hurricane Protection 						     10

20			Mailboxes, Posts and Exterior Light Posts			     10

21			Flagpoles							     10

22			Private Wells							     10

23			Sidewalks							     11

24			Outdoor Cooking, Smokers and Fire Pits			     11

Berkshire Lakes BLMA, Inc. 
Architectural Review Board Standards

1	ARB Authorization

The Architectural Review Board (ARB) is established under the direction of the Berkshire Lakes BLMA, Inc.(“BLMA”)  Article 7 of the Fourth Amended and Restated Declaration of Covenants, Conditions, Restrictions and Easements for Berkshire Lakes outlines the make-up, scope and responsibilities of the ARB.  The ARB shall have the right to require the submission of site, elevation, landscape, irrigation, and/or construction plans.

1.1	ARB approval is necessary prior to commencement of any work on the exterior of any residential unit.  Failure to obtain such approval may result in, among other things, undoing the work at the Residential Unit Owner’s expense.

1.2	As part of the ARB Alteration Request application process, 1 complete set of plans (the ARB may, in its discretion, require up to 3 sets of plans), drawings/diagrams, photographs, and specifications prepared by the Residential Unit Owner, an architect, landscape architect, engineer or other person found to be qualified by the ARB shall be submitted for approval by written application on such forms as may be provided or required by the ARB. When submitting a landscape plan, either new or revised, the plan must include the location of any street lamp located on or near the residential unit.

1.3	Complete plans must be submitted to the ARB before applying to Collier County for any permit(s).

1.4	Plans incident to said work shall, in the sole discretion of the ARB, be complete and accurately depict the improvements being contemplated. All changes involving design, color or materials must be resubmitted for approval prior to commencing work. All work shall comply with ARB approvals.

1.5	It shall be the Residential Unit Owner’s obligation to obtain all required Collier County permits.

1.6	It is the Residential Unit Owner’s responsibility to ascertain if and where any and all easements are located on his or her property and be aware of all restrictions within those easements.

1.7	The ARB has the right to approve or disapprove any request on an individual basis.  Building lot size, location, and orientation can be the determining factors in the approval process for plantings, fences and hedges.  The standards established by the ARB may be altered or amended from time to time as the ARB determines, subject to BLMA Board approval.

1.8	Any request for an addition or alteration to an Residential Unit Owner’s property that is not listed within this document needs to be submitted to the ARB.  The ARB will review the request and it will need BLMA Board approval.

1.9	ARB approval does not override governmental regulations.  ARB approval does not override decisions made by the HOA governing documents in regard to drainage, grading or clearing of easements.

2	ARB Alteration Request Process

  - Step1:  Residential Unit Owner completes an ARB Alteration Request form.

  - Step 2:  Form is received, checked for completeness including resident supplied projected start and completion dates.

  - Step 3:  Sequential project number is assigned and information is logged in the ARB Alteration Request tracking log.

  - Step 4:  At the next regularly scheduled ARB meeting the request shall be evaluated and if there are no objections, the resident will receive an approval and be permitted to implement the request.  If the request is not approved, the Residential Unit Owner cannot move forward with the project. Residential Unit Owners can resubmit paperwork with any clarifications or changes to seek approval.

  - Step 5:  After Collier County approves, the Residential Unit Owner may commence work.

  - Step 6:  Upon issuance of a final inspection approval by Collier County, Residential Unit Owner shall notify the office that the project has been completed and return the permit (yellow card).

  - Step 7:  The property shall be inspected by a representative of BLMA, to ensure the completed project complies with the initial request.

  - Step 8:  After the inspection the resident shall receive a written notice of approval or disapproval from BLMA.

3	Definitions

The following definitions apply the Berkshire Lakes Architectural Review Board Standards:

3.1	Deck:  Flooring surrounding an in-ground pool or spa, or floor of a patio, lanai, or porch.  It must be constructed of pavers or concrete.

3.2	Fence: A barrier intended to prevent escape or intrusion or to mark a boundary.  It typically consists of posts and slats.  No wood, chain-link or wire fences are allowed.

3.3	Hedge:  A boundary formed by closely growing bushes or shrubs of the same species.

3.4	Lanai:  A roofed porch.

3.5	Porch:  A covered area adjoining an entry.

3.6	Screening Materials:  Concrete, decorative concrete, PVC, or plantings.

3.7	Structure:  Anything constructed or erected with a fixed location on the ground. Among other things, these may include buildings, walls, fences, and swing sets.

3.8	Wall: A thick masonry structure.

3.9	Thicket:  An area of dense or uncontrolled growth of trees, bushes, and/or underbrush.

4	Contractor/Residential Unit Responsibilities

4.1	Unless the Residential Unit Owner and its contractor receives specific written permission from the ARB, the Residential Unit Owner and its contractor and all other persons involved in the construction of a residential improvement shall perform their work without entering upon property owned by any Residential Unit Owner or entity other than the person or entity that owns the property upon which the residential improvement is being constructed.

4.2	Contractors or any other person providing labor services and materials shall cause any vehicle or equipment, materials, or debris to be placed upon property owned by a Residential Unit Owner or entity other than the person or entity that owns the property upon which the residential improvement is being constructed.

4.3	Residential Unit Owners of the property upon which the residential improvement is being constructed shall be responsible for any violations of this standard and if any damage is caused to adjacent or nearby property, or if any debris is placed upon adjacent or nearby property in violation of this standard, the Residential Unit Owner of the property upon which the residential improvement is being constructed shall be liable for all actual and consequential damages occasioned thereby. In all cases, all construction debris must be removed from the construction site no later than 5 working days after any Certificate of Occupancy or Certificate of Completion is issued by Collier County.

4.4	Dig Safe (call 811 or online at www.sunshine811.com) must be contacted to mark all underground utilities prior to any work being performed.  It is the Residential Unit Owner’s responsibility to protect and maintain all utilities.  Any damages must be repaired immediately by the Residential Unit Owner and his/her contractor.

4.5	BLMA shall have the right to require any construction to cease, if such construction is not in compliance with the approval given by the ARB.  Neither the BLMA nor the ARB shall be liable for any damages related to ceasing such construction.

5	Drainage and Grading

5.1	Drainage and grading shall meet all requirements of the proper government agencies

5.2	The contractor shall grade in accordance with the overall drainage plan and other criteria as may be required by the ARB.

5.3	Collier County has adopted a Surface Water Drainage Plan for the Berkshire Lakes subdivision within and upon the rear 5 feet of each of the following-described single family residential lots. There exists a nonexclusive surface water drainage easement that runs in favor of BLMA and in favor of the following Residential Unit Owners of any properties within Berkshire Lakes that the plan shows as utilizing said easement for surface water drainage:

Lot		Street Address			Lot		Street Address

12		744 Belville 				107		809 Belville

13		748 Belville 				108		865 Belville

14		752 Belville				121		761 Belville

15		756 Belville				125		723 Lambton

20		637 Merryport				27		696 Lambton

95		849 Belville				90		261 Lambton

96		845 Belville				91		267 Lambton

99		6637 Eastbourne			158 		703 Lambton

100		6641 Eastbourne			161		6621 Cutty Sark

103		6640 Eastbourne			30		656 Lambton

104		6636 Eastbourne			62		645 Lambton

- This easement shall entitle the BLMA to grade and slope the easement area so that surface waters flow over the easement area in accordance with the plan adopted by Collier County, South Florida Water Management District and, where applicable, the Department of Environmental Protection.

- The depth and slope of the easement area shall be in accordance with the plan adopted by Collier County, South Florida Water Management District and, where applicable, the Department of Environmental Protection.

- The aforementioned easement includes the right to drain surface water from other residential lots over and upon easement areas, provided that such drainage is in accordance with the plan.

6	Landscaping

Before modifications in a landscape plan are made, the landscape plans reflecting said modifications shall be submitted for approval to the ARB.  This does not include annual plantings.

The ARB was established to preserve the beauty, quality and value of the Berkshire Lakes Community. If a Residential Unit Owner chooses to implement the States’s voluntary Florida Friendly Landscaping practices, it must be implemented without disruption of the overall consistent appearance of the community or negatively impact property values. The ARB Board has evaluated the standards and approved changes that can be applied without disrupting the cohesive look of our deed restricted community.

All approved landscaping must be properly maintained.  If it is deemed by the BLMA to have become unsightly, the owner will be required to correct the landscaping issue by trimming or removing it.

6.1	Healthy, living tree removal must have ARB approval.  The Residential Unit Owner is responsible to contact Collier County to determine tree requirement.

6.2	All plantings must not extend on to any neighboring property at maturity. See https://gardeningsolutions.ifas.ufl.edu/ to reference maturity size of plantings.

6.3	Ficus and running/spreading bamboo as well as any invasive species/County Prohibited Exotics are banned.

6.4	All lots are required to be sodded and have an automatic underground irrigation system installed.

6.5	The Residential Unit Owner shall sod and irrigate the entire lot up to the street valley gutter, up to the high-water mark for lakeside lots and adjacent to all landscaped and or natural areas which border the individual building lot.

6.6	All grasses and ground covers must adhere to Berkshire Lakes existing standards for lawns (referred to as sod). It must be green in color year-round and properly maintained by Residential Unit Owner.

6.7	A 5 foot strip of sod shall be provided adjacent to the house side of all sidewalks, where present, or along roadways where sidewalks are not present.

6.8	10 feet of sod surrounding all lakes must be maintained, extending to the high-water mark of the lake.

6.9	Rock/mulch is to be contained in defined planting beds.  No rock/ mulch strips without plants along driveways for the purpose of parking. All rock/mulching to the curb, sidewalk or driveway must have proper edging in order to contain the rock/mulch and soil below from being washed into the drains/sewers which impacts the lakes.

6.10	No rock/mulching of the entire property.  Our covenants require sod/ARB approved artificial turf on all lots.

6.11	Fertilization of turf and plantings must follow the Collier County fertilization ordinance.

6.12	Vegetable and fruit gardens are not permitted in the front of any property.  A vegetable or fruit garden may be permitted in the side yard if it is not visible from the street.

6.13	Thickets are not considered proper landscaping in Berkshire Lakes.  All landscaping must be properly maintained and controlled.

Landscaping Adjacent to Common Properties

6.13	Maintenance responsibility for all common properties, which includes the entire exterior of the perimeter fence, is assigned to the BLMA.

6.14	Planting trees/hedges, and shrubs within the 10-foot easement on the interior of the perimeter fence is prohibited. Doing so hampers BLMA’s ability to maintain/repair the fence as required.

6.15	Should the Residential Unit Owner fail to abide by the above restriction and a repair/maintenance activity requires close access to the fence, BLMA is not liable for damages to said plantings. In such event, the Residential Unit Owner shall be liable.

7	Fences

No chain link, wire, wood or wooden stockade fence of any kind is permitted. (See appendix)

Fences replacing pool cages are not required to have a hedge screening the fence.  All other fences must have a landscape buffer between the fence and adjacent property.  If a neighboring property has an existing landscape buffer which shields your fence from their view, you are not required to plant an additional buffer.  However, if at any time in the future the landscape buffer is removed, you are required to install a buffer to shield the fence from view of the adjacent property.

Building lot sizes can be the determining factor in the approval process for fences to be constructed in the space between adjacent residential units.  Factors considered by the ARB are the specific space in feet between the residential units, the impact on the adjacent Residential Unit Residential Unit Owner’s ability to maintain his/her property, and the ability for both Residential Unit Residential Unit Owners to access their property.

Pool Area:

- Fences in lieu of enclosures of swimming pools, must be 4 feet in height.  The BLMA Community pool is excluded from this requirement for security reasons.

- Fences replacing pool cages are not required to have a hedge screening the fence.

Perimeter (Yards facing Santa Barbara Blvd., Radio Rd., St. Clair Shores Rd., Tina Ln., I-75, and the Golden Gate Canal):

  - Perimeter areas of the development may have up to a 9-foot fence on the section that is facing the perimeter.

Front of Homes:

  - No fence shall be permitted in the front of any property.

Between Homes:

  - Fences must be a minimum of 5 feet from the property line.  This is to leave room for the planting and maintenance of a hedge to conceal the fence from view.

The properties of 252 Lambton, 256 Lambton, 929 Belville, and 7201 Appleby along the bicycle/walking trail have had 6-foot wooden fences to screen the backyard from view.  If these are replaced, they must be constructed of ARB approved materials.  Replacement fencing can be the original height and cover the area in which the fence was originally constructed, but cannot exceed it.

8	Hedges

The hedge on both sides must be kept trimmed by the Residential Unit Owner, so as not to encroach on any neighbor’s property.  The Residential Unit Residential Unit Owner must be able to keep the hedge trimmed without trespassing on the neighboring property.

All hedges shall be set back a minimum of 3 feet from the property line.  The ARB may require more than the minimum 3-foot setback on hedges between two homes if it is deemed to interfere with access to the backyard area.

Hedges installed on the property line prior to April 20, 2005 are exempt from the aforementioned setbacks; however, the Residential Unit Owner(s) are responsible for the maintenance.  Dead or otherwise destroyed hedges cannot be replaced on the property line and desired replacements are subject to the ARB approval process.

The use of Areca Palms as a hedge between adjacent properties is prohibited as of November 2009, and cannot be replaced.

Building lot sizes are the determining factor in the approval process for hedges to be constructed in the space between adjacent houses.  Factors to be considered are the specific space in feet between the homes, the impact on the adjacent property Residential Unit Owner’s ability to maintain his/her property, and the ability for both Residential Unit Owners to access their property.

Pool Area:

  - Fences replacing pool cages are not required to have a hedge screening the fence.

Perimeter (Yards facing Santa Barbara Blvd., Radio Rd., St. Clair Shores Rd., Tina Ln., I-75, and the Golden Gate Canal):

  - Perimeter areas of the development may have up to a 9-foot hedge on the section that is facing the perimeter.

  - Areca Palms can be utilized to form a noise barrier; however, they must be planted 10 feet from the perimeter fence to allow maintenance access. Examples are:

BLMA’s use of the Areca Palm to screen the wooden fence at the rear of the Lambton Lane Park. BLMA’s use of the Areca Palm along the I-75 corridor to develop a noise barrier.

Residential Unit Residential Unit Owner’s use of the Areca Palm to screen fences and or create a noise barrier along the Santa Barbara Blvd. and Radio Rd. corridors, along the canal facing I-75(Not planted within the canal easement), and those residential units backing up to the perimeter fence.

Front of Homes:

- Hedges installed in the front yard will be set back a minimum of 10 feet from the edge of the street.

- A hedge should not exceed 4 feet if planted in the front yard unless it is planted against the wall of the home where it can be a maximum of 6 feet in height. The ARB has the right to approve heights above these limits for properties (see section 1.7).  Exceeding these height requirements must be specifically approved by the ARB.

Between Homes:

- A hedge shall not exceed 6 feet in height between adjacent residential units.

For lake front lots only, the hedge continuing to the rear property line, once past the patio area must be reduced in height to 4 feet.

- To maintain a uniform height of a hedge, the 6 feet around all sides of the deck or patio is measured from the top of the deck or patio.

9	Walls

Walls must follow the same height and setback rules as fences.

When using walls to create small privacy gardens, they must be designed to coordinate with the overall design of the house.

10	Shoreline

At the time of lake remediation, most lots which abut the lakes will have had sod installed at the seasonal high-water mark above a course of stone which continues down the slope into the lake itself. No riding lawn mowers are permitted to operate within five feet of the lake edge of the sod. No fertilizer should be applied within ten feet of the lake edge. The ARB does not take applications for any changes to the shoreline.  Any changes must be addressed to the BLMA.  Nothing other than sod may be planted in the 10 to 15 foot easement granted to the BLMA for shoreline maintenance and repair.

11	Mechanical Equipment

11.1	A/C units and pool pump, filter and heating equipment, water softeners, solar equipment and associated electrical panels and controls must be screened from view from road and neighbors where possible.

11.2	Equipment should be located for minimum noise to occupant and neighbors where possible.

12	Accessory Structures

Carports and tool sheds are prohibited.

13	Swimming Pools, Spas and Hot Tubs

13.1	Residential Unit may include an attached or separate spa.

13.2	Swimming pools and spas are to be below ground/deck level.  A spa attached to the pool may be above grade.

13.3	Hot tubs shall be within the boundaries of any screened enclosure or if not located within a screened lanai, hot tubs shall have a locking mechanism for the top, or be located within a fenced area. No surrounding wood or metal decks.  A landscape buffer must screen a hot tub from view.

13.4	Pool, spa, or lanai enclosures shall be bronze or white (no “raw” aluminum)

14	Lanai and/or Patio Roof

14.1	Lanais and patio roofs must be attached to the Residential Unit, or no further than 2 feet from the Residential Unit’s roof.

14.2	Lanais and patio roofs must be professionally installed and hurricane rated.

14.3	A lanai, patio, or patio roof which may be added after the original residential unit’s construction, must be consistent with the design and material of the original residential unit.

14.4	Flat aluminum roofs are acceptable if must flat/parallel to ground with a slight pitch where neighbors cannot see the roofing materials, otherwise roofing must be constructed from approved materials (See roofing section 16)

14.5	All aluminum roofs must have facia/trim caps/covers so as to shield edges from view.

14.6	The color of all aluminum patio coverings must be bronze or white (no "raw" aluminum)

15	House Painting

All homes should be painted every 5-10 years.  It is a Residential Unit Owner’s responsibility to properly clean and paint homes on a regular basis. Recommended colors and color combinations are contained in the ARB color book which may be reviewed prior to application. Paint samples must be included with ARB Request form.

Owners must submit a drawing of where the colors will be painted, as well as a photograph of the home’s exterior.  Color swatches must be submitted with the application to paint the home.  No photocopies or electronic submissions of colors will be approved (as they are often not true to the actual color).

Acceptable Wall/Main Paint Colors:

- Neutral colors such as white, beige and gray, and most pastel/lighter colors are acceptable.

- No pink, purple or brown wall color.

- No bright blues or greens may be painted on the main walls.

- Certain dark colors may be appropriate, but must be submitted to the ARB.

Acceptable Trim, Accent, and Garage Door Colors:

- All colors must coordinate with wall and roof colors.

- Trim and Accent colors may be contrasting with wall color, but must coordinate with the wall and roof colors.

- The ARB may approve colors not listed in the color book for homes with white or off-white walls.

- The ARB may limit the number of colors painted on a home.

Acceptable Front Door Colors:

- Bright or dark front door colors that may not be appropriate for wall colors may be approved if they coordinate well with the rest of the home’s color scheme.

Acceptable Driveway Colors:

- Neutral colors such as white, beige and gray are acceptable.

- Driveway colors must coordinate well with the rest of the home’s color scheme.

Acceptable Hurricane Shutter Colors:

- Neutral colors such as white, beige and gray.

- All colors must coordinate with wall and roof colors.

- If shutters are permanent and decorative (such as Bahama shutters) they will fall under the restrictions listed in the acceptable trim, accent or garage door colors section.

16	Driveways

16.2 	A Collier County permit shall be obtained for any driveway alterations.

16.3	Driveways and turnarounds may be limited in size or placement in the discretion of the ARB depending on the lot size and orientation.

16.4	Driveways shall be constructed of concrete or pavers.  They may be stamped, stained, or painted.  Asphalt pavement or gravel driveways are not permitted.

16.5	Driveway colors must be approved.  Refer to the acceptable color guidelines.

17	Roof Materials

17.1 	Clay, cement tile, metal and asphalt shingles are the acceptable roof materials.

18	Garage Door Screens

Garage door screens are permitted when adhering to the following guidelines:

- They are overhead mounted either under the soffit or attached directly to the garage door header beam.

- The screen must be one piece and cannot have an entrance door.

- Screens may be electrically or manually operated.

- The screen material must be of the “SOLAR” type, which ensures that the garage interior is screened from public view.

19	Hurricane Protection

Residential Unit Owners may utilize the following methods to protect their units during hurricanes:

- Roof systems recognized by the Florida Building Code which meet ASCE 7-22 standards

- Permanent fixed storm shutters

- Roll-down track storm shutters

- Impact-resistant windows and doors

- Polycarbonate panels

- Reinforced garage doors

- Erosion controls

- Exterior fixed generators

- Fuel storage tanks not visible to neighboring properties

- Other hurricane protection products used to preserve and protect the structures or improvements on a parcel governed by the association

19.2 	Any hurricane protection method utilized by a Residential Unit Owner will comply with all Florida Building Codes.

19.3	Any hurricane protection method shall conform to the unified building scheme of BLMA regarding external appearance of the structure.  A color sample must be approved by the ARB.

20	 Mailboxes, Posts and Exterior Light Posts

20.1	Residential Units must have black mailboxes and posts with post light fixtures. Mailboxes must have house numbers a minimum of 1 inch tall.

Mail-post installation measurements

21	Flags/Flagpoles

21.1	A home may have 1 free-standing flagpole no more than 20 feet high.

21.2	The flagpole cannot obstruct sightlines at intersections or interfere with light from street lamps, and cannot be erected within or upon an easement.

21.3	The flagpole and display are subject to all building codes, zoning setbacks, and other applicable government regulations, including lighting ordinances.

21.4	In lieu of a flag pole the Residential Unit Owner can elect to install a wall mounted flagpole assembly.

22	Private Wells

22.1	Any private well improvements placed upon a residential unit are placed thereon at the sole risk of the Residential Unit Owner.

22.2	All approvals granted by the ARB may be revoked at any time if the ARB determines that the actual operation of a well is causing staining to buildings and or other improvements, is causing unpleasant odors, is creating unacceptable noise or is adversely affecting residential unit or Residential Unit Owner.  The Residential Unit Owner shall remove of any such staining.

23    	Sidewalks

23.1	Sidewalks where installed are part of the public right-of-way. As such repairs are the responsibility of Collier County.

23.2	Sidewalk cleaning of mold, dirt and litter shall be performed by the Residential Unit Owner whose residential unit is adjacent to the sidewalk.

24	Outdoor Cooking, Smokers, and Fire Pits

24.1	Only commercially manufactured electric, LP gas, or charcoal outdoor portable barbeque appliances are permitted.

24.2	Commercially manufactured electric or LP gas outdoor portable fire pit/heaters are permitted.

24.3	No portable barbecue grills or portable fire pits/heaters shall be of a size, or used in a location or manner, that presents a danger to the Residential Unit Owner or other persons, or unreasonably intrudes upon other Residential Unit Owner’s use and enjoyment of their residential unit.  The use of such devices is restricted to the backyard.

24.4	No permanent outdoor barbeque grill, fire pit, or fireplace structures of any kind are permitted.

24.5	No outdoor wood burning devices of any kind are permitted, including but not limited to wood-burning fireplaces, fire pits, barbeque grills, and food smoking devices.

Residence Requirements:

Minimum Floor Area
Air-conditioned floor area excludes garage, entry, and lanai or porch area

1.0 	One story 1200 square feet

1.1	Two story 1400 square feet.

Front Elevations

1.0	Consideration by the contractor should be given to avoid construction of similar            elevations close to each other.

1.1 	Any proposed building should be in harmony with the architecture of buildings on the neighboring properties.

1.2	Where practical, natural materials, rather than synthetic will be used for exterior finish.

1.3	Wing wall additions to the structure shall not be higher than 5 feet above the slab.

Roof Pitch

1.1 	Main structure and garage - minimum center pitch 5 feet high to 12 feet horizontal. 2.2 Lanai - if part of the original design plan, the lanai may have a flat or below the 3½ minimum pitch roof

Single family Residential 90' and 70' lots
Permitted uses:

| 1.0 Single Family Dwelling 1.1 Building Setbacks: as measured from individual lot or parcel boundary lines.1.1.1 Lot size | 70.0' | 90.0' |
| --- | --- | --- |
| 1.1 Minimum front yard | 25.0' | 30.0' |
| 1.2 Minimum side yard |  |  |
| One story | 7.5' | 7.5' |
| Two Story | 7.5' | 10.0' |
| 1.3 Minimum Rear Yard |  |  |
| Principal Structure | 20.0' | 25.0' |
| Accessory Structure | 10.0' | 15.0' |

Maximum Building Height

2.0	30 feet, principal structure

2.1	20 feet, garage and pool enclosure

2.3	As measured from the finished floor to the highest point of the roof structure.

Minimum Floor Area

1.0	Air conditioned floor area excludes garage, entry, lanai or porch area.

1.1	One story - 1800 square feet.

1.2	Two story - 2000 square feet.

1.3	All lots platted in Unit 7 Berkshire Lakes will be required to have

a minimum floor area for a one story structure of 1500 square feet.

Accessory Structures

1.0	Private garages - with overhead doors - may be front or side entry design.

1.1	2 car attached – minimum

1.2   	No maximum attached garage

1.3	 Remote electric automatic door opener on each door is required.

1.4	Carports and tool sheds are prohibited.

Private Swimming Pools, SPA

1.0	May include an attached, or detached, separate spa

1.1	Pool Enclosure Color - bronze or white (no "raw" aluminum)

1.3 	Swimming pools and spas are to be below ground or deck level.

Single Family Residential — 55' Lot
With Collier County site plan approval, Villas, cluster and group housing, townhouses, patio homes and zero lot line homes.

Building Setbacks
As measured from individual lot or parcel boundary lines.

1.0	Lot size - 55'

1.1	Minimum Front Yard	20'

1.2	Minimum Side Yard - One Story	5'		Two Story	5’

1.3	Minimum Rear Yard -Principal Structure 20'

1.4	Accessory Structure 5'

Maximum Building Height

1.0	30 feet, principal structure

1.1	20 feet, garage & pool enclosure

1.2	As measured from the finished floor to the highest point of roof structure.

Accessory Structures

1.0	Private garages - with overhead doors - may be front or side entry design.

1.1	1 car attached — minimum

1.2	No maximum attached garage

1.3	Remote electric automatic door is required for each door.

Corner lot residences are to face minor streets where possible.